The table below outlines the environmental regulations for ships operating or transiting within the Firth of Forth, and Firth of Tay

| Activity | Regulation |
| --- | --- |
| Underwater cleaning of hull. | Not permitted unless growth can be captured and disposed of on land as waste. |
| Hull painting. | Not permitted in port. |
| Paint chipping | Not permitted in port. Vessels may chip at anchor or while underway outside of port. |
| Fuel restrictions | As per MARPOL Annex VI. |
| Use of incinerators | As per MARPOL Annex VI. |
| Grey water discharge | Not permitted in enclosed dock. Permitted in tidal waters including the estuary, Crombie, Rosyth and other estuarial marine terminals if the grey water is treated in appropriate plant as per MEPC 227 (64). |
| Discharge of garbage | As per MARPOL Annex V. and Merchant Shipping (Prevention of Pollution by Garbage from Ships) Regulations 2020 For individual port waste management plans see: https://www.forthports.co.uk/marine/information/. |
| Discharge of treated blackwater/sewage | As Per MARPOL Annex IV, Merchant Shipping (prevention of pollution by sewage and garbage from ships) regulations 2008 and 2010 |
| Discharge of pool/spa water | Fresh water and salt water discharge permitted only. |
| Discharge of scrubber water | Any discharge to water is strictly not permitted. |
| Discharge of commutated food waste | As per MARPOL Annex V. For individual port waste management plans see: https://www.forthports.co.uk/marine/information/. |
| Rinsing of the hull | Fresh or salt water only. |
| Propeller polishing | Dependent on level and origin of marine growth, and methodology of cleaning. Contact FTNS to request permission. |
| Discharge of advanced wastewater purification treated blackwater | As per MARPOL Annex IV. |
| Discharge of ballast water | As per IMO International Convention for the Control and Management of Ship’s Ballast Water and Sediments. and Merchant Shipping (Control and Management of Ships’ Ballast Water and Sediments). |
| Venting/gas freeing | Not permitted in the Forth Ports Statutory Harbour Authority area. Information can be found at https://www.forthports.co.uk/wp-content/uploads/2023/08/Mariners-Guide-Forth-2023.pdf section 19. |
| Bunkering | As per Forth Ports Bunkering Procedure (Scotland): https://www.forthports.co.uk/marine/information/ |
| Pollution | Any pollution from a vessel or sighted must be reported immediately to: Forth & Tay Navigation Service (FTNS) T: +44 (0)132 4498 584 (24/7) E: ftns@forthports.co.uk VHF CH: 71 Call sign: Forth Navigation |